How to Capture Images from the Oscilloscope using BenchLink Software.

(Adapted from notes first written by Prof. Maurice Aburdene)

- On your computer, go to Start > Programs > Programs A-H > BenchLink > HP 34810B BenchLink Scope.

- Once the software loads, a warning message appears.  Select ‘Ok’.

- A ‘Setup Interface’ window should now appear.  For the ‘Interface’ select ‘National Instruments AT-GPIB’ or ‘GPIB-PC2’ instead of ‘RS232’.  A window should appear with the title ‘Interface Change’.  Click ‘Yes’ to prompt the computer to search for the GPIB interface and the instrument connected to that interface.

- The instrument(s) window should display information about the oscilloscope.  Select the instrument and click ‘Ok’.

- In the BenchLink program select Image > New… , and answer ‘Ok’ when the window appears notifying you that the program is about to capture a screen image.  This step will capture the image on the oscilloscope and display it on the computer’s screen.

- Select File > Save as… to save an image file.  The files are saved with a .pcx extension.  You can paste or insert these files into a Word document.

- To capture waveforms from individual channels, select Waveform Data > New… .  A window will appear asking you to select one or more channels.  Press ‘Ok’ when you are ready to capture the data.